EXPERIMENT NO. 03

SIMULATION AND ANALYSIS OF 2-INPUT CMOS NAND AND NOR GATES

―――――――――――――――――――――――――――――――――――――――――――――――――――――――

AIM

To simulate and analyze the schematics and transient waveforms of 2-input CMOS NAND and NOR gates using LT Spice.

SOFTWARE USED

LT Spice

CIRCUIT DIAGRAM

The schematic diagrams for 2-input NAND and NOR gates:

Figure 3.1: LT Spice Schematic of 2-Input CMOS NAND Gate

Figure 3.2: LT Spice Schematic of 2-Input CMOS NOR Gate

Transistor architecture of 2-input CMOS NAND gate:

Figure 3.3: CMOS Two-Input NAND Gate Structure

SMALL THEORY

NAND and NOR gates are universal logic gates realized using static complementary CMOS logic, comprising a Pull-Up Network (PUN) of PMOS transistors and a Pull-Down Network (PDN) of NMOS transistors.

1. 2-Input CMOS NAND Gate:

Circuit Architecture: PUN consists of two PMOS transistors (Q1, Q2) connected in parallel between VDD and output. PDN consists of two NMOS transistors (Q3, Q4) connected in series between output and ground.

Operation: Output is LOW (0) only when both inputs A and B are HIGH (1), activating both series NMOS devices. When either input is LOW (0), at least one parallel PMOS conducts, pulling the output HIGH (1).

Truth Table for 2-Input NAND Gate:

| Input A | Input B | Output Y (NAND) |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

2. 2-Input CMOS NOR Gate:

Circuit Architecture: PUN consists of two PMOS transistors connected in series between VDD and output. PDN consists of two NMOS transistors connected in parallel between output and ground.

Operation: Output is HIGH (1) only when both inputs A and B are LOW (0), conducting through both series PMOS devices. When either input is HIGH (1), a parallel NMOS conducts, pulling the output LOW (0).

Truth Table for 2-Input NOR Gate:

| Input A | Input B | Output Y (NOR) |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

OUTPUT GRAPH

Simulated transient waveforms from LT Spice:

Figure 3.4: Simulated Output Waveforms of 2-Input CMOS NAND Gate

Figure 3.5: Simulated Output Waveforms of 2-Input CMOS NOR Gate